Final project- single cycle CPU

Compen331 Section002

Yiyi Wang

Xiaodong Huo

4/28/18

Abstract

This project is a summery to previous labs. This concludes contents from lab4,lab5 and lab6. Instruction that can execute includes instruction fetch, instruction execution, R-format instruction, I-format instruction, Branch instructions, Jump instructions, and shift instructions. Then we need to generate bit-stream and connect to board. When all the instructions are done(pc=h5c), led should light up.

Introduction

There are five stages including instruction fetch, decode, execution, memory access and writeback.

In the instruction fetch state, a program counter reads the address of the instruction and then access the Instmem. The value of the program counter is determined by the 2bit pcsrc. Determined by the instruction type. Then a 32 bit instruction is decomposed based on the instruction type.

In the decode stage, op code and function code is sent to the control unit, Rs and Rt is send to the Regfile. A mux determines the the destination register number rt or rd base on the type of instruction(rt if regrt = 1). A mux f determines the j-type instructions takes the jump address. If jal is a 1, a 5 bit patern 11111 (decimal 31) will be assigned to the destination register number wn. Otherwise, reg_dest, which is rd or rt, will be assigned to wn. The mux below the f mux determines the destination address, it will be either Program counter+4 or the alu output memory or the output of the data memory. When the instruction is branch type, a imm address will be sign extended and the immediate is sign extended

(if sext  =  1), after the sign extension, a shift left logic with shift the address to the left and add whatever on the PC+4. Input 3 is the jump target address coming from addr and PC+4.

The output of regfile is qa and qb correspond to rna and rnb.

In the excution state, two mux determines one operand of the addition comes from register rs of the register file (shift = 0); the other operand is the immediate (aluimm = 1) for immediate operations. In the alu, the alu control determines the type of operations excuted. It can be add, subtract, and, or, xor, lui, sll,srl, sra based on the 4bit aluc code.

In the Memory access stage, we determines the write enable, whether to write in the memory or not.

In the write back stage, the wreg singla controls whether to write back to the regfile or not.

The benefits of our design is we separated the data memory and instruction memory so that we can read and write from the memory at the same time.(read at positive edge and write at negative edge)

RTL analysis

I/O planning

Floor Planning

Zybo

TOP Execute

`timescale 1ns / 1ps

module execute(clk, clrn, led);

input clk, clrn;

wire [31:0] instruc;

wire [31:0] r;

wire [31:0] pc, next_pc;

wire m2reg, wmem, shift, aluimm, wreg, regrt, sext,z;

wire [3:0] aluc;

wire [1:0] pcsrc;

wire [4:0] wn, wn_new;

wire [31:0] d, d_new, do, qa, qb, a, b, imm_exp, imm_exp_shift, addr_shift, p1, p2, addr_exp, jal;

output led;

pc PC(clk, clrn, pc,next_pc);

read_instruc readInstruc(next_pc, instruc);

pc_adder p1Adder(next_pc,32'd4, p1);

addr_e addrE(instruc[25:0], p1[31:28], addr_shift);

imm_e immE(sext, instruc[15:0], imm_exp);

shift_left immShift(imm_exp, imm_exp_shift);

pc_adder p2Adder(p1, imm_exp_shift, p2);

control_unit controlUnit(instruc[31:26], instruc[5:0],z, m2reg, wmem, aluimm, pcsrc, aluc, shift, wreg, regrt, sext, jal);

regrt_mux regrtMux(regrt, instruc[15:11], instruc[20:16], wn);

wn_mux wnMux(jal, wn, wn_new);

d_mux dMux(jal,d, p1, d_new);

regfile regFile(clk, clrn, wreg, instruc[25:21],instruc[20:16], wn_new, d_new, qa, qb);

pc_mux pcMux(pcsrc, p1, p2, qa, addr_shift, pc);

shift_mux shiftMux(shift, qa, instruc[10:6], a);

qb_imm_mux qbImmMux(aluimm, qb, imm_exp, b);

ALU alu(aluc, a, b, r, z);

data_mem dataMem(clk, clrn, wmem, r, qb, do);

m2reg_mux m2regMux(m2reg, r, do, d);

//assign led = !(next_pc == pc)? 0:1;

assign led = !(next_pc >= 32'h0000005c) ? 0:1;

//assign led = clrn;

endmodule

Test Bench

`timescale 1ns / 1ps

module test_bench;

reg clk = 0;

reg clrn;

wire led;

execute PCexecute(clk, clrn,led);

always #10 clk = ~clk;

initial begin

#0 clk = 0;

clrn = 0;

#15 clrn = 1;

#1350 $finish;

end

endmodule

PC

`timescale 1ns / 1ps

module pc(clk, clrn, input_pc, next_pc);

input clk;

input clrn;

input [31:0] input_pc;

output reg [31:0] next_pc;

//initial begin

//    next_pc <= 32'b0;

//end

always @(posedge clk) begin

if (clrn) begin

next_pc <= input_pc;

end

else begin

next_pc <= 32'h00000000;

end

end

endmodule

Read instruction

`timescale 1ns / 1ps

module read_instruc(pc, instruc);

input [31:0] pc;

output reg [31:0] instruc;

reg [31:0] instruc_mem [0:31];

initial begin

instruc_mem[0] <= 32'h3c010000;

instruc_mem[1] <= 32'h34240050;

instruc_mem[2] <= 32'h20050004;

instruc_mem[3] <= 32'h0c000018;

instruc_mem[4] <= 32'hac820000;

instruc_mem[5] <= 32'h8c890000;

instruc_mem[6] <= 32'h01244022;

instruc_mem[7] <= 32'h20050003;

instruc_mem[8] <= 32'h20a5ffff;

instruc_mem[9] <= 32'h34a8ffff;

instruc_mem[10] <= 32'h39085555;

instruc_mem[11] <= 32'h2009ffff;

instruc_mem[12] <= 32'h312affff;

instruc_mem[13] <= 32'h01493025;

instruc_mem[14] <= 32'h01494026;

instruc_mem[15] <= 32'h01463824;

instruc_mem[16] <= 32'h10a00001;

instruc_mem[17] <= 32'h08000008;

instruc_mem[18] <= 32'h2005ffff;

instruc_mem[19] <= 32'h000543c0;

instruc_mem[20] <= 32'h00084400;

instruc_mem[21] <= 32'h00084403;

instruc_mem[22] <= 32'h000843c2;

instruc_mem[23] <= 32'h08000017;

instruc_mem[24] <= 32'h00004020;

instruc_mem[25] <= 32'h8c890000;

instruc_mem[26] <= 32'h20840004;

instruc_mem[27] <= 32'h01094020;

instruc_mem[28] <= 32'h20a5ffff;

instruc_mem[29] <= 32'h14a0fffb;

instruc_mem[30] <= 32'h00081000;

instruc_mem[31] <= 32'h03e00008;

end

always @(*) begin

instruc <= instruc_mem[pc>>2];

end

endmodule

P1 Adder

`timescale 1ns / 1ps

module pc_adder(current_pc, adder, next_pc);

input [31:0] current_pc;

input [31:0] adder;

output reg [31:0] next_pc;

always @(*) begin

next_pc <= current_pc + adder;

end

endmodule

Adder

`timescale 1ns / 1ps

module addr_e(addr, pc, addr_exp);

input [25:0] addr;

input [3:0] pc;

output [31:0] addr_exp;

assign addr_exp[31:28] = pc;

assign addr_exp[27:2] = addr;

assign addr_exp[1:0] = 2'b00;

endmodule

Imme

`timescale 1ns / 1ps

module imm_e(sext, imm, imm_exp);

input sext;

input [15:0] imm;

output reg [31:0] imm_exp;

always @(*) begin

if (sext == 1) begin

if (imm[15] == 0) begin

imm_exp[31:16] <= 16'b0;

imm_exp[15:0] <= imm;

end

else begin

imm_exp[31:16] <= 16'hffff;

imm_exp[15:0] <= imm;

end

end

else begin

imm_exp[31:16] <= 16'b0;

imm_exp[15:0] <= imm;

end

end

endmodule

Immshift

`timescale 1ns / 1ps

module shift_left(data_in, data_out);

input [31:0] data_in;

output reg [31:0] data_out;

always @(*) begin

data_out <= (data_in<<2) & 32'hffffffff;

end

endmodule

P2 adder

`timescale 1ns / 1ps

module pc_adder(current_pc, adder, next_pc);

input [31:0] current_pc;

input [31:0] adder;

output reg [31:0] next_pc;

always @(*) begin

next_pc <= current_pc + adder;

end

endmodule

Control unit

`timescale 1ns / 1ps

module control_unit(op, func, z, m2reg, wmem, aluimm, pcsrc, aluc, shift, wreg, regrt, sext, jal);

input [5:0] op;

input [5:0] func;

input z;

output reg m2reg;

output reg wmem;

output reg aluimm;

output reg [1:0] pcsrc;

output reg [3:0] aluc;

output reg shift;

output reg wreg;

output reg regrt;

output reg sext;

output reg jal;

always @(*) begin

if (op == 6'b000000)

begin

if (func == 6'b100000)

begin                 // add

pcsrc <= 2'b00;

aluc <= 4'bx000;

shift <= 0;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 0;

regrt <= 0;

jal <= 0;

end

else if (func == 6'b100010) begin // subtract

pcsrc <= 2'b00;

aluc <= 4'bx100;

shift <= 0;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 0;

regrt <= 0;

jal <= 0;

end

else if (func == 6'b100100) begin // and

pcsrc <= 2'b00;

aluc <= 4'bx001;

shift <= 0;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 0;

regrt <= 0;

jal <= 0;

end

else if (func == 6'b100101) begin // or

pcsrc <= 2'b00;

aluc <= 4'bx101;

shift <= 0;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 0;

regrt <= 0;

jal <= 0;

end

else if (func == 6'b100110) begin // xor

pcsrc = 2'b00;

aluc = 4'bx010;

shift = 0;

wreg = 0;

m2reg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 0;

regrt <= 0;

jal <= 0;

end

else if (func == 6'b000000) begin  // sll

pcsrc <= 2'b00;

aluc <= 4'b0011;

shift <= 1;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 0;

regrt <= 0;

jal <= 0;

end

else if (func == 6'b000010) begin // srl

pcsrc <= 2'b00;

aluc <= 4'b0111;

shift <= 1;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 0;

regrt <= 0;

jal <= 0;

end

else if (func == 6'b000011) begin  // sra

pcsrc <= 2'b00;

aluc <= 4'b1111;

shift <= 1;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 0;

regrt <= 0;

jal <= 0;

end

else if (func == 6'b001000) begin  //jr

pcsrc <= 2'b10;

wmem <= 0;

wreg <= 0;

end

end

else if (op == 6'b001000) begin  // addi

pcsrc[1:0] <= 2'b00;

aluc <= 4'bx000;

shift <= 0;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 1;

sext <= 1;

regrt <= 1;

jal <= 0;

end

else if (op == 6'b001100) begin  // andi

pcsrc[1:0] <= 2'b00;

aluc <= 4'bx001;

shift <= 0;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 1;

sext <= 0;

regrt <= 1;

jal <= 0;

end

else if (op == 6'b001101) begin  // ori

pcsrc[1:0] <= 2'b00;

aluc <= 4'bx101;

shift <= 0;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 1;

sext <= 0;

regrt <= 1;

jal <= 0;

end

else if (op == 6'b001110) begin  // xori

pcsrc[1:0] <= 2'b00;

aluc <= 4'bx010;

shift <= 0;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 1;

sext <= 0;

regrt <= 1;

jal <= 0;

end

else if (op == 6'b100011) begin  // lw

pcsrc[1:0] <= 2'b00;

aluc <= 4'bx000;

shift <= 0;

wreg <= 1;

m2reg <= 1;

wmem <= 0;

aluimm <= 1;

sext <= 1;

regrt <= 1;

jal <= 0;

end

else if (op == 6'b101011) begin  // sw

pcsrc[1:0] <= 2'b00;

aluc<= 4'bx000;

shift <= 0;

wreg <= 0;

wmem <= 1;

aluimm <= 1;

sext <= 1;

end

else if (op == 6'b000100) begin  // beq

if (z == 0) begin  // z = 0

pcsrc[1:0] <= 2'b00;

aluc <= 4'bx010;

shift <= 0;

wreg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 1;

end

else begin  // z = 1

pcsrc[1:0] <= 2'b01;

aluc <= 4'bx010;

shift <= 0;

wreg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 1;

end

end

else if (op == 6'b000101) begin  // bne

if (z == 0) begin  // z = 0

pcsrc[1:0] <= 2'b01;

aluc <= 4'bx010;

shift <= 0;

wreg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 1;

end

else begin  // z = 1

pcsrc[1:0] <= 2'b00;

aluc <= 4'bx010;

shift <= 0;

wreg <= 0;

wmem <= 0;

aluimm <= 0;

sext <= 1;

end

end

else if (op == 6'b001111) begin  // lui

pcsrc[1:0] <= 2'b00;

aluc <= 4'bx110;

wreg <= 1;

m2reg <= 0;

wmem <= 0;

aluimm <= 1;

regrt <= 1;

jal <= 0;

end

else if (op == 6'b000010) begin  // j

pcsrc[1:0] <= 2'b11;

wreg <= 0;

wmem <= 0;

end

else if (op == 6'b000011) begin  // jal

pcsrc[1:0] <= 2'b11;

wreg <= 1;

wmem <= 0;

jal <= 1;

end

end

endmodule

`timescale 1ns / 1ps

RegrtMux

module regrt_mux(regrt, rd, rt, wn);

input regrt;

input [4:0] rd;

input [4:0] rt;

output reg [4:0] wn;

always @(*) begin

if (regrt == 1)

wn <= rt;

else

wn <= rd;

end

endmodule

wnMux

`timescale 1ns / 1ps

module wn_mux(jal, wn_in, wn_out);

input jal;

input [4:0] wn_in;

output reg [4:0] wn_out;

always @(*) begin

if (jal == 0)

wn_out <= wn_in;

else

wn_out <= 5'b11111;

end

endmodule

Dmux

`timescale 1ns / 1ps

module d_mux(jal, d_in, pcPlus4, d_out);

input jal;

input [31:0] d_in, pcPlus4;

output reg [31:0] d_out;

always @(*) begin

if (jal == 0)

d_out <= d_in;

else

d_out <= pcPlus4;

end

endmodule

Refile

`timescale 1ns / 1ps

module regfile(clk,clrn, wreg, rna, rnb, wn, d, qa, qb);

input clk;

input clrn;

input wreg;

input [4:0] rna;

input [4:0] rnb;

input [4:0] wn;

input [31:0] d;

output [31:0] qa, qb;

reg[31:0] register [0:31];

assign qa = (rna == 0)? 0:register[rna];

assign qb = (rna == 0)? 0:register[rnb];

integer i;

//initial begin

//    for (i=0; i<32; i=i+1)

//        register[i] <= 32'b0;

//    end

always @(posedge clk) begin

if (clrn == 0)begin

for (i=0; i<32; i=i+1) begin

register[i] <= 32'b0;

end

end

if ((wn != 0) && (wreg == 1))

register[wn] <= d;

end

endmodule

PC mux

`timescale 1ns / 1ps

module pc_mux(pcsrc, pc_1, pc_2, pc_3, pc_4, next_pc);

input [1:0] pcsrc;

input [31:0] pc_1;

input [31:0] pc_2;

input [31:0] pc_3;

input [31:0] pc_4;

output reg[31:0] next_pc;

always @(*) begin

case (pcsrc)

'b00: next_pc = pc_1;

'b01: next_pc = pc_2;

'b10: next_pc = pc_3;

'b11: next_pc = pc_4;

endcase

end

endmodule

Shift Mux

`timescale 1ns / 1ps

module shift_mux(shift, qa, sa, a);

input shift;

input [31:0] qa;

input [4:0] sa;

output reg [31:0] a;

always @(qa or sa or shift)

begin

if (shift)

begin

a[31:5] <= 27'b0;

a[4:0] <= sa;

end

else

a <= qa;

end

endmodule

qblmmMux

`timescale 1ns / 1ps

module qb_imm_mux(aluimm, qb, imm, b);

input aluimm;

input [31:0] qb;

input [31:0] imm;

output reg [31:0] b;

always @(*) begin

if (aluimm == 1)

b <= imm;

else

b <= qb;

end

endmodule

ALU

`timescale 1ns / 1ps

module ALU(aluc, a, b, r, z);

input [3:0] aluc;

input [31:0] a;

input [31:0] b;

output reg [31:0] r;

output reg z;

integer i;

reg [31:0] x;

always @(aluc, a, b)

begin

if (aluc[2:0] == 3'b000) // add

r <= a + b;

else if (aluc[2:0] == 3'b100) //sub

r <= a - b;

else if (aluc[2:0] == 3'b001) //and

r <= a & b;

else if (aluc[2:0] == 3'b101) // or

r <= a | b;

else if (aluc[2:0] == 3'b010) begin //xor

r <= a ^ b;

if (a == b)

z <= 1;

else

z <= 0;

end

else if (aluc[2:0] == 3'b110) begin // lui

r[31:16] <= b[15:0];

r[15:0] <= 16'b0;

end

else if (aluc == 4'b0011) // sll

r <= b << a;

else if (aluc == 4'b0111) // srl

r <= b >> a;

else if (aluc == 4'b1111) begin // sra

r <= b >>> a;

end

end

endmodule

Data Mem

`timescale 1ns / 1ps

module data_mem(clk, clrn, wmem, a, di, do);

input clk, clrn, wmem;

input [31:0] a;

input [31:0] di;

output [31:0] do;

reg [31:0] dataMem [0:31];

integer i;

assign do = dataMem[a>>2];

always @(posedge clk)begin

if(clrn == 0) begin

for (i = 0; i < 32; i = i + 1) begin

dataMem[i] = 0;

end

dataMem[5'h14] = 32'h000000a3;

dataMem[5'h15] = 32'h00000027;

dataMem[5'h16] = 32'h00000079;

dataMem[5'h17] = 32'h00000115;

end

else begin

if (wmem == 1) begin

dataMem[a>>2] <= di;

end

end

end

endmodule

m2regMux

`timescale 1ns / 1ps

module m2reg_mux(m2reg, r, do, d);

input m2reg;

input [31:0] r;

input [31:0] do;

output reg [31:0] d;

always @(*) begin

if (m2reg == 1)

d <= do;

else

d <= r;

end

endmodule

Constraint

##Clock signal

set_property -dict { PACKAGE_PIN L16   IOSTANDARD LVCMOS33 } [get_ports { clk }]; #IO_L11P_T1_SRCC_35 Sch=sysclk

create_clock -add -name sys_clk_pin -period 8.00 -waveform {0 4} [get_ports { clk }];

##Switches

set_property -dict { PACKAGE_PIN G15   IOSTANDARD LVCMOS33 } [get_ports { clrn }]; #IO_L19N_T3_VREF_35 Sch=SW0

##LEDs

set_property -dict { PACKAGE_PIN M14   IOSTANDARD LVCMOS33 } [get_ports { led }]; #IO_L23P_T3_35 Sch=LED0